# Tri-Power Installation Instructions

# Tri-Power motors with Smart Power offer the easiest and most flexible method for wiring motorized shades. It is important to follow these installation steps for a successful installation.

# Parts/Tools Needed:

- 18/3 or 18/4 wire

- Wire strippers

- 3-conductor splitters (one per motor-part# H18-CONN-3WAY)

- Smart Power motor cables (one per motor-part# H18-USB-6FT or H18-USB-12FT)

- Tri-Power bus tester (part# H18-TESTER).

- Step 1: Locate a good place for the Smart Power Hub

  - This should be determined early in construction in order to have power in the correct place

  - Must have a 120V outlet for power

  - Try to locate the Smart Power Hub towards the middle of the installation to maximize wire length

  - Each bus connection is capable of powering up to 20 motors on up to 500’ of 18 AWG wire

- Step 2: Pull 18/3 or 18/4 wire to the window locations from the Smart Power Hub location (18/4 will give you an extra conductor should you find a short later on)

  - Wire can be pulled in any configuration including straight line, star pattern, tree pattern, or any combination of those as long as the 500’ accumulative wire length is met.

  - Use the 3-conductor splitters to branch off in different directions if needed

  - Make sure to leave 6”-12” of extra wire at motor locations for trimming and stripping wire in the next step

- Step 3: Cut and strip the wire at motor locations

  - Using wire strippers, strip back the outer coating of the wire to expose the inner conductors

  - Strip back the inner conductors to expose 3/8” to 1/2” of bare wire

- Step 4: Install Splitters to one side of the bus

  - Connect one of the bus terminals on the Smart Power Hub to the bus wires by connecting the red wire to the red connection, the black wire to the black connection, and the white wire to the white connection

  - Starting at the closest motor location to the Smart Power Hub, connect one side of the splitter only

  - On the splitter connect the red wire to the red connection, the black wire to the black connection, and the white wire to the white connection

- Step 5: Power up and test the bus

  - Power up the Smart Power Hub by plugging into a power outlet

  - Connect the first motor cable to the splitter by connecting the red wire to the red connection, the black wire to the black connection, and the white wire to the white connection

  - Plug the motor cable USB-C end into the Bus Tester

    - The green and yellow LED on means bus is good

    - Other LED lights mean one of the following:

      - red and black only - only green LED on

      - white wire to red terminal - only green LED on

      - white wire to black terminal - only green LED on

      - red wire and black wire reversed - only red LED

  - Connect the next leg of the bus to the splitter and repeat “step 5” for all motor locations working one motor at a time working away from the Smart Power Hub

- Step 6: Install the shades and plug in motors

  - Using the motor cables plug in one motor at a time working from the motor closest to the Smart Power Hub away

  - Once a motor is plugged in to the motor cable you will see a blink sequence of blue-yellow-blue yellow. This confirms everything is functioning properly

    - If there is no LED blink on the motor check for conductivity using the bus tester

- Step 7: Program remotes and/or keypads to motors

  - Add keypads and/or remotes by using the programming instructions included in the keypad or remote boxes